TU/CODL

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (SPRING) 2020

DRE 201: ENERGY MANAGEMENT AND AUDITING

Time: 3 hours                                   Full Marks: 70

The figures in the right-hand margin indicate marks

for the individual questions

***************************************************

- Choose the correct option:                                                    2x10=20

- The sector with the lowest commercial energy consumption out of four major sectors in the country is-

i. Industry       			ii. Agriculture 
   iii. Residential   	               iv. Transport

- Energy manager should be well versed with-

- Managerial and technical skills

- Managerial and marketing skills

- Technical and economical analysis skills

- Energy audit skills

- Which of the following is least important of a good energy information system?

- data accuracy

- data relevance

- data volume

- data validation

- The ratio of energy consumption to corresponding production is called-

- specific energy consumption

- Energy intensity

- production energy ratio

- specific production ratio

- Energy supplied by combustion of fuel is equal to-

- fuel mass x density

- fuel mass x specific heat

- fuel mass x calorific value

- fuel mass x heat capacity

P.T.O.

- A three-phase induction motor is drawing 12 amps at 440 volts. If the operating power factor of the motor is 0.85, then the power drawn by the motor is-

- 7.8 kW

- 4.5 Kw

- 5.3 kW

- 23.4 kW

- The sum of present values of all cash flows associated with a project called-

- NPV

- PV

- Internal rate of return

- return on investment

- An energy audit requires-

- Quantification of energy use

- Qualification of energy use

- Converting all energy use to one single unit

- Reduction in power consumption

- A firm substitutes (replaces) expensive oil and starts firing

wood in a boiler. The following scenario is most likely-

- Higher energy consumption

- higher fuel cost

- better boiler efficiency

- less smoke in the stack

- Amount of oxygen (stoichiometric) needed for complete combustion of 1 kg of Methane

- 17.39 kg

- 4 kg

- 6 kg

- None of the above

2. Answer the following:                                                            6+4+5=15

- Explain: Energy management, energy conservation and energy auditing.

- Explain the difference between energy conservation and energy efficiency and state one example where energy costs are reduced but energy consumption goes up.

- Mention any three responsibilities and duties of an energy manager.

3. Answer the following questions:                               3+6+3+4+4=20

- Explain the use of sankey diagram.

- Installation of Rs 25,000 energy management systems in an industry is expected to results in a 25% reduction in electricity and 40% savings in process heating costs. These translate to net yearly savings of Rs 6,500 and Rs. 8,500 respectively. If the energy management system has an expected useful life of 8 years, determine the Net Present Value of the investment. Salvage value need not be considered in the analysis and discount rate is 12%.

- What is the significance of calculating specific energy consumption (SEC)?

- Discuss four energy efficiency improvement opportunities in a boiler system.

- What is waste heat? How do you convert the waste heat to useful form of heat?

4. Answer the following questions:                                    3+3+3+6=15

- Define the power factor of an electrical gadget.

- How the power factor improvement is related with the energy conservation programme?

- How does power factor of an induction motor reduce with the reduction of the applied load on the motor?

- A tube light is rated 40W, 230V and 50Hz, it draws a current of 0.3 A. Calculate (i) power factor and (ii) size of the capacitance bank required for improving the existing power factor to 0.95.

***